# Capstone Design Project Report

Starlab Payload Power System

Team 16

Greyson Donley

Fatima Sayeed

Lucas Brickner

Electrical and Computer Engineering Department

College of Engineering

Ohio State University

ECE 3906

04-27-2026

Document Change Notice

| Date | Change |
| --- | --- |
| 12/12/2025 | Initial Release for review |
| 4/10/2026 | Chapter 3 |
| 4/20/2026 | Update Appendix B |
| 4/27/2026 | Chapters 4,5,6 |

Executive Summary

The scope of this project was to create a power system that can supply voltages of 120VAC, 120VDC, and 28VDC to various payloads. The input of the system was 120VDC at 2.4kW. The system would also be able to monitor the power being drawn by a payload. Due to delays in communication and time restraints, the scope of the project was changed to focus on the 28VDC voltage level for one payload. Since this change in scope occurred later in the team's design process, the current sensor PCB is still designed to read all three voltage levels. The power of the system is measured using a Raspberry Pi 3B+ communicating with a Pico 2 connected to the current sensor PCB. These two microcontrollers communicate through I2C. The Pico 2 measures the power through the current sensors located on the same PCB. They communicate through SPI. The voltage and current are measured every 0.5 seconds to ensure real-time, accurate data. The current prototype consists of a 120VDC to 28VDC converter to achieve the desired 28V output.  The output of the converter is connected to two adjustable DC/DC converters. One is set to 12V to power the cooling fan located near the three converters. The other converter is unused in the current prototype but would be used to power the Raspberry Pi and current sensors.

The designed prototype was tested following the outlined requirements from the earlier stages of this project. The requirements included acceptable temperature, noise, size, and power range. The current design as a whole and the subsystems function well within the temperature range of 18-26 ˚C; the cooling fan helps in preventing overheating of the converter. The prototype also remains within the noise range of 25-64 dB in 1 meter. When tested, the prototype produced 61 dB at 1 meter. Furthermore, the entire design is 457.2 x 457.2 x 161.93 mm, which ensures it is within the size requirement of 535 x 600 x 195 mm; the product is compact in nature. Additionally, the power requirement of 500W could not be tested properly due to power supply limitations. However, the converter is rated up to 600 W and it was tested up to 347W.

In the remaining chapters of this report, the prototype’s final design and justifications are included. Additionally, the two-step communication and its algorithms are explored in detail in the latter portion of this body of work. There are detailed analyses on the cost of designing this prototype and projected costs for accommodating the original scope of this project. This report also includes a user guide and troubleshooting guide to ensure users are able to navigate this design with ease. The team has explained potential areas of work for future teams and suggestions for navigating this project.

Table of Contents

Chapter 1: Problem Definition and Preliminary Design	1

1.1 Problem Statement	1

1.2 Stakeholders and End Users	1

1.3 Customer Needs, Projects Constraints, and Requirements	2

1.4 Use of Engineering Standards	3

1.5 Social, Environmental, and/or Global Issues Impact	4

1.6 Research and Market Study	4

1.7 Preliminary Design	5

1.7.1 Block Diagram:	5

1.7.2 Converter Selection:	6

1.7.3 Microcontroller Selection:	7

Chapter 2: Detail Design	9

2.1 Design Concepts	9

2.2 System Functionality	10

2.3 Power Monitoring	10

2.4 Power Conversion	11

Chapter 3: Prototype and Testing	13

3.1 Prototype	13

3.2 Testing	16

Chapter 4: Final Design	20

Chapter 5: User Manual	32

Chapter 6: Project Management, Summary, and Conclusion	36

6.1 Project Management	36

6.2 Summary	36

6.3 Conclusion	37

Figures and Tables

Figure 1: Initial design concept. One voltage level for one payload.	5

Figure 2: Initial design concept, Six voltage levels for one payload	6

Figure 3: Converter Schematic, 120 VDC to 28 VDC	6

Figure 4: Converter Simulation. Black: 28V, Blue: 16V, Red: 5V.	7

Figure 5: Individual switch vs rotary switch control	9

Figure 6: 120VDC to 28VDC Branch	11

Figure 7: 120VDC to 120VAC Branch	12

Figure 8: 120VDC Branch.	12

Figure 9: Updated system diagram with two 28V payloads.	13

Figure 10: Current sensor PCB with Pico 2 attached using headers.	14

Figure 11: Schematic for connecting 28VDC sensor to a load for monitoring.	14

Figure 12: 120VDC-28VDC converter on the custom PCB.	15

Figure 13: All components on the aluminum mounting plate.	16

Figure 14: Final Prototype	20

Figure 15: Final Prototype Block Diagram	21

Figure 16: Sensor PCB connection	22

Figure 17: Sensor PCB design	23

Figure 18: Sensor PCB version 2. Modified from the original to use a Pico 2 as well as using JST connectors for more convenient use.	23

Figure 19: 28VDC sensor connections for PCB	24

Figure 20: 28VDC sensor voltage measurement accuracy.	24

Figure 21: 28VDC sensor current measurement accuracy.	25

Figure 22: 28VDC Converter PCB Design.	25

Figure 23: 28VDC Converter PCB Design.	26

Table 1: Project Stakeholder Analysis	1

Table 2: Project Requirements	3

Table 3: Requirement/Verification Cross-Reference Matrix	17

Table 4: Prototype Three Voltage Levels Cost	30

Table 5: Cost Per Payload	31

# Chapter 1: Problem Definition and Preliminary Design

## 1.1 Problem Statement

Airbus and Voyager Technologies are developing a new low Earth orbiting (LEO) station, Starlab, to replace the aging International Space Station (ISS). Here, scientific research will be conducted to study high-vacuum and low-gravity environments. To utilize the equipment the astronauts will need, a payload workbench is being commissioned. This workbench serves as a mounting point and power source for the payloads. The team is tasked with designing and building the power distribution and control system for the workbench. From here, the power will be converted into five levels of DC voltage as well as one level of AC voltage. Power consumption at each output will be monitored to provide information to the crew and ground control.

## 1.2 Stakeholders and End Users

Prior to any design of this workbench, it is imperative to understand the stakeholders’ needs for this project. A project’s success is not solely its functionality; rather, it comes from functionality and satisfying the specific needs of stakeholders. Table 1 describes our stakeholders, their specific needs, and the extent of their involvement in this project.

Table 1: Project Stakeholder Analysis

| Name | Department / Company | Position | Advisers | Objectives, Requirements, Interests | Influence | Project Contribution | Resistance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Michael Jentsch & Florian Strigl | Airbus | Lead Engineers | Lead Engineers | Customer needs a functioning power system that can power 10 payloads. System can be monitored on an active interface. | High | Requirements | Low - Initiated Project |
| Dr. Zia | OSU ECE Department | Faculty Advisor | Capstone Director | Successful capstone project will allow for future opportunities. | Medium | Technical mentorship, academic guidance | Low - supports student success |
| Dr. Horack | OSU AERO & MECH Department | Vice President of Research | Main Sponsor | A high-quality outcome will solidify the relationship between the university and Airbus. | High | High level management | Medium- product functionality and meets customer needs |
| John Trzinski | OSU Starlab | Payload System Manager | Technical Advisor | Functioning power system to ensure a successful workbench | Medium | General Scope and Feedback | High- specific requirement for project functionality |

Some of the major stakeholders of this project are the lead engineers from Airbus, Michael Jentsch and Florian Strigl. Their ultimate requirement from the team is to generate a functional power system capable of powering ten payloads. Additionally, the system must be able to report power usage at each output. They have a high-level influence on this project as they are involved in outlining specific requirements. Another major stakeholder is the Electrical and Computer Engineering (ECE) Capstone Director, Dr. Saideh Zia. The main objective of Dr. Zia is that the team delivers a successful project adhering to all necessary engineering principles. This will help ensure future Capstone students are presented with more opportunities. Although she has a medium-level influence on the project’s technical aspect, Dr. Zia’s involvement is to provide technical and academic mentorship in the process.

The project’s main sponsor is the university’s Vice President of Research, Dr. John Horack. His involvement in the project is very minimal. However, he is responsible for high-level management, thus having the greatest degree of influence. The team’s success in this project will play a part in ensuring a better relationship between the university and Airbus. Finally, the payload system manager of the Ohio State Starlab team, John Triznski, is one of the stakeholders. His ultimate requirement for the team is to provide a functional power system. He is responsible for guiding the team with feedback and establishing the general scope.

## 1.3 Customer Needs, Projects Constraints, and Requirements

The workbench will generally be managed by astronauts and individuals with intermediate electrical experience. Therefore, the designed power system must be easy to maintain and operate. As a way of catering to those needs, the power system is designed to operate on the Inter-Integrated Circuit (I2C) protocol.  The implementation of the I2C protocol allows for power monitoring without manual intervention. Furthermore, the workbench has manual switches at each voltage level, which serve as an intuitive method for the crew to control the outputs.

In addition to customer needs, there are key requirements that the power system must abide by to ensure its safety and usability. Table 2 demonstrates the five specific criteria and their ranges that make the workbench safe and operational. The workbench must function in an environment of 18-26°C. The ideal sound emitted by the workbench should be 25 dB at 1 m so that it is as quiet as realistically possible. The maximum continuous noise level of 64dB is a requirement set by NASA for 60 Hz frequencies [1, Table 3.7.2.2-1]. The size and power wattage requirements are based on the guidelines from the preliminary design provided by Airbus [2].

The designed power system will be running continuously. Therefore, extra precautions may be needed to protect the system from overheating. An overheated power system would not only function poorly but could also pose a safety risk. The air-cooling system of the payload rack will aid in controlling temperatures in the power system compartment. Heat sinks can be installed on the current sensors and buck converters to further improve temperatures as needed. All system components are checked for thermal capabilities to avoid performance issues.

Regarding project constraints, two major factors are time and budget. Due to a delayed meeting with sponsors, the team lacked information throughout their research phase, ultimately delaying the project timeline. Additionally, certain necessary components are difficult to source and exceed the budget limit. The team has reevaluated the project timeline to ensure the power system is complete in the remaining time in the spring.

Table 2: Project Requirements

| Needs | Requirement | Unit | Range (acceptable) | Ideal |
| --- | --- | --- | --- | --- |
| Thermal Operation Range | Able to operate in expected conditions | °C | 18-26°C | 26°C |
| Noise level | Low noise during operation | dB | 25-64 dB at 1 m | 25 dB |
| Size | Fit in allocated space in rack | mm3 | 535 x 195 x 600 mm | 535 x 195 x 400 mm |
| Power | Support full use of all payloads | W | 2400W total 500W at 28VDC 2000W at 120VDC 600W at 120VAC | 2400W total 500W at 28VDC 2000W at 120VDC 600W at 120VAC |

## 1.4 Use of Engineering Standards

Engineering Standards play a critical role in guiding the design choices made during the project. Engineering standards ensure that the power system for the Starlab payload workbench is safe, reliable, and capable of delivering power in an environment where an electrical failure can be disastrous. The National Electric Code (NEC) forms a set of requirements for wire gauge sizes and overcurrent protection. These rules affected the decisions that were made in the design of the project, such as wire selection, breaker sizes, and safe distribution of power to the payloads [3].

Another area of safety concern is the custom PCB design for the current sensors. These boards will have to withstand high power draw for extended periods of time. IPC2152 specifies how a PCB should be designed. The most relevant requirement that the team will follow is the calculation for trace widths and spacing. High currents and voltages require larger trace areas and spacing. IPC2152 also categorizes PCBs into three classes. Due to the project being in the aerospace industry, the design must be fail-proof to be used in Starlab. Therefore, the PCB design will need to be the most stringent class, class three.

Engineering Standards also affect component selection. During the decision-making process, the components were selected using data sheets to ensure the product was being used according to the manufacturer’s specifications. Factors like voltage and current ratings were used to select power converters, current sense resistors, and toggle switches to ensure the system could sustain expected power requirements. Manufacturers such as Texas Instruments (TI) follow many regulatory and voluntary standards when designing and creating their products. Some of these include ANSI, IEEE. These standards establish confidence in the industry that any of the products produced by TI and other manufacturers will perform as stated by the datasheet. By following these standards, the final power system for Starlab’s payload racks will be safe for use in its intended applications.

## 1.5 Social, Environmental, and/or Global Issues Impact

The nature of this project is niche to the area of electronics and space. The impact of this power system will solely befall on the operators and the interacting systems. It is highly improbable for this project to be sensitive to any demographic or establish grounds for potential lawsuits. Similar to other methods of space exploration, Starlab and Airbus will act in accordance with international space laws and regulations. Furthermore, the designed power system will adhere to the rules and regulations of electronics and engineering standards; details of such regulations are previously outlined in subsection 1.4.

It is worth noting the design for Starlab draws inspiration from the existing International Space Station (ISS). In fact, the team’s research involved understanding the workings of the EXPRESS racks on the ISS. The designed power system is inspired and enhanced from past examples. While the teams' design for Starlab is unique, it is able to accommodate heritage payloads from the ISS. Backwards compatibility aims to reduce waste by allowing existing payloads from the ISS to continue to be utilized in Starlab. This eliminates potential electronic waste.

## 1.6 Research and Market Study

The research conducted for this project is focused on safe distribution of the input of 120VDC along with controlling and monitoring the power at each output. Research started by looking at existing power technology used in the ISS to better understand what we need to achieve from this power system [4], [5]. The preliminary design from Airbus was also a source of information on the plan for the Starlab payload rack. Based on this research, the Exeltech 120VDC-120VAC inverter was chosen. This inverter is used on the ISS and was recommended to be used again. The high input voltage resulted in this inverter meant Exeltech was the only viable option.

Market research was conducted on which power converters would be able to support the 120VDC to 28VDC conversion at the desired 600W. Multiple vendors such as Digi-Key, Allied Electronics, and Mouser Electronics were searched to compare pricing and availability. Due to the short timeline of the project, in-stock parts were exclusively chosen.

The 120VDC-28VDC converter was chosen due it is appropriate power rating and lowest price for a readily available buck converter. Some other potential options were the TDK-Lambda PAF500F48-28 which was close to the desired power rating, but the input voltage was too low. Resistor divider circuits could have been Another potential power converter was the 150V Vicor Maxi family. This power converter fits the power needs to be selected but was not chosen due to having a substantially higher price.

Research on power monitoring led to using shunt resistors. The resistor in the current sensor creates a small voltage drop that is dependent on the current. This voltage difference allows for calculation of the power being used by that output. After this, the information needs to be communicated back to the microcontroller (MCU). The selected current sensor uses the I2C protocol to do this. I2C uses a pair of wires to send information and can support many devices connected in series. The INA745BIRELR was selected to monitor the 28VDC 500W output due to the products rating and cheaper cost. The current resistor for the 128VDC line is still to be selected due to higher voltage and power losses from the added resistance.

The microcontroller must be able to read the data coming from the current resistor and display the information. A Raspberry Pi 3 B+ was selected using the decision matrix located on Figure 6 in Section 1.7.  Using weighted scores, the raspberry pi outperformed the other four microcontrollers with factors like cost, I2C access, CPU speed, Memory, and I/O types. The microcontroller is discussed further in section 1.7.3.

The switches and circuit breakers were selected based on their power ratings stated on the datasheet. In accordance with the NEC 2023 regulations, all circuit breakers are sized to be 25% over the expected maximum amperage. [3] This leaves room for slight current fluctuations as well as not pushing the breaker to its thermal limits.

## 1.7 Preliminary Design

### 1.7.1 Block Diagram:

Figure 1 illustrates the general design of the power system for a single payload running on a single voltage level. A connection from the voltage source to the buck converter is established to set the voltage level. Then, it connects to a circuit breaker for overcurrent protection. The converted voltage now goes through the current sensor. The sensor and the microcontroller have two connections to communicate over I2C. The manual switch opens/closes the circuit to the payload, and the LED serves as a status reminder. Figure 2 shows how the same design can expand for multiple voltage levels. It is worth noting that since the team’s Preliminary Design Review with sponsors, the team was informed that lower voltage levels are not of priority. Thus, the focus remains on 120 VDC, 120 VAC, and 28 VDC.

Figure 1: Initial design concept. One voltage level for one payload.

Figure 2: Initial design concept, Six voltage levels for one payload

### 1.7.2 Converter Selection:

The performance of each converter was analyzed through simulation to check functionality. Figure 3 shows the schematic for 120 VDC to 28 VDC conversion. Each converter was simulated in a similar fashion. Figure 4 shows each converters’ performance in comparison. Voltage levels 16, 12, and 5 must be disregarded.

Figure 3: Converter Schematic, 120 VDC to 28 VDC

Figure 4: Converter Simulation. Black: 28V, Blue: 16V, Red: 5V.

### 1.7.3 Microcontroller Selection:

In order to select the best functioning microcontroller for the power system, several microcontrollers were evaluated based on five different criteria - Cost, I2C Connection, CPU Speed, Memory, and I/O Types.  A weight was chosen for each category to prioritize relevant characteristics. A decision matrix was used to select the ideal microcontroller as seen in Appendix A. Amongst the options, the Raspberry Pi 3 B+ was shown to be the best choice with a score of 13.8.

References

[1] NASA, EXPEDite the PRocessing of Experiments to Space Station (EXPRESS) Rack Payloads Interface Definition Document, SSP 52000-IDD-ERP, Rev. H, Sept. 2009.

[2] M. Jentsch, S. Stenzel, M. Myers, J. Kissling, and E. Pucek, “Starlab Subsystem PDR Internal Payload Laboratory”, Oct. 7, 2025.

[3] National Fire Protection Association, NFPA 70: National Electrical Code, 2023 ed., Quincy, MA: NFPA, 2022.

[4] T. B. Davis, J. B. Adams, E. M. Fisher Jr., G. B. Prickett, and T. G. Smith, "EXPRESS Rack Technology for Space Station," presented at a technical conference, 1999.

[5] E. B. Gietl, E. W. Gholdston, B. A. Manners, and R. A. Delventhal, "The Electric Power System of the International Space Station—A Platform for Power Technology Development,"
[6] Boeing, International Space Station Electric Power System (EPS), St. Louis, MO, technical publication, n.d.

[7] NASA, International Space Station Payload Accommodations Handbook: EXPEDite the PRocessing of Experiments to Space Station (EXPRESS) Rack Payloads, SSP 52000-PAH-ERP, Initial Release, Nov. 24, 1997.

[8] NASA Tech. Memo. 2000-210209, presented at the IEEE Aerospace Conference, Big Sky, MT, Mar. 18–25, 2000.

# Chapter 2: Detail Design

## 2.1 Design Concepts

Based on the initial concept discussed in section 1.7, two primary designs were created as seen in Figure 5. The core of the system is the same for each concept. The key difference lies in the control of the outputs. One option is to use a dedicated switch for each output. This method is intuitive for the crew. However, it will drive project costs up due to needing many switches rated for high voltages. Additionally, the large number of switches will congest the crew interface.

Another option is to use one rotary switch to control all outputs for a given payload. The blue circle in Figure 5 indicates where the rotary knob would go. The knob would connect 28VDC to a relay to turn on the selected output. This method would greatly reduce the number of switches but would not allow for multiple outputs to be on at once. After consulting with the Airbus team, the individual toggle switches were chosen to maintain flexibility of the power system.

Figure 5: Individual switch vs rotary switch control

## 2.2 System Functionality

The power source from Starlab’s solar panels and battery banks provides 2400W at 120VDC. Converters and inverters are used to step down voltage to each level needed. Circuit breakers are connected in each output. This protects the payloads along with the power system components. Current sensors use a small shunt resistor to measure current. The small voltage drop causes a power loss, but the resistor is sized to keep the loss minimal. The toggle switches with a LED indicator clearly show the crew which outputs are on.

This concept is easily expanded to as many payloads as needed. Each payload would branch off after voltage step-down. New circuit breakers, current sensors, and switches would be needed for each payload expansion. The team aimed to keep the system as modular as possible. This makes maintenance simple along with having the ability to switch an output to a new voltage in the future.

## 2.3 Power Monitoring

To monitor the power draw on each output, the Texas Instruments INA745 current sensor is used. This sensor works by measuring the voltage drop across an 800µΩ resistor. With a maximum current of 20A, the largest voltage drop is 0.016V, which causes a loss of 0.32W. With 600W available, the loss of 0.05% power was determined to be acceptable. The information the sensor collects is communicated to the microcontroller using the I2C protocol. This protocol was chosen for its ability to link all the sensors on the same bus. This means only one pair of wires will be needed to communicate with all sensors.

The I2C protocol implements a master-slave relationship between the microcontroller and the current sensors using two signals. The Serial Data line (SDA) is used for transferring data which, in this scenario, are the current and the voltage measurements. The Serial Clock line is used to send clock signals from the Raspberry Pi to the sensors. The microcontroller initiates communication, and the sensors respond accordingly. The protocol follows a simple algorithm - allocation of registers and defining parameters, configuring the sensor to the appropriate registers, reading in and appropriate conversion of the acquired values, and finally outputting the calculated values. The defined algorithm was programmed using Python for efficiency. The team chose to read information from the current sensors every three seconds as a compromise between updated data and power usage. The frequency the data is read at can be easily adjusted in the Raspberry Pi’s code.

Hall effect sensors are an alternate option to the shunt resistor sensor INA745. Hall effect sensors use the magnetic field generated by the current to calculate the current through the line without adding a resistance. Thus, making them a great choice to avoid unnecessary power loss. The tradeoff is that hall effect provides lower accuracy. The team is evaluating if eliminating the power loss is worth the decrease in accuracy.

## 2.4 Power Conversion

A CFB600W-110S28 DC-DC converter is used to step down the 120VDC input to 28VDC. The converter can take a range of 43-160VDC as an input and can output a maximum of 21.4A. This corresponds to a maximum output power of 600W. Both the input and output use 12 AWG wiring to support the 20A current. The 28 VDC output of the converter runs through a 1019986-circuit breaker rated at 25A. This ensures the breaker does not trip under full load at 20A while still providing protection from overcurrent. After the breaker, the power is measured by the INA745BIRELR current sense resistor which monitors power being drawn by the payload. The power then passes through a S333 toggle switch. This switch controls the power that will be delivered to the payload. The switch is rated for 30VDC at 25A to make sure it can handle this branch’s power. The MCU requires a 5V power supply at 3A. This power supply comes from the TPS629333DRLR converter, which has an input range of 3.3-30VDC and provides a 3A output current. The resistors surrounding the chip set the output to 5V. A 3.3V supply is needed to power the current sensors. This is created by the same TPS629333DRLR converter, except the resistors are selected to set the output at 3.3V. The schematic for this branch is found in Figure 6.

Figure 6: 120VDC to 28VDC Branch

Figure 7 shows the schematic for the 120VAC output. A DC-AC inverter is used to get the 120VAC output. The XP6-1-I-6-1 takes the 120VDC input and outputs 120VAC at 60Hz. The output current is 5A, so the power will be 600W. This branch is protected by a 2907484-circuit breaker rated at 7A. After the breaker, the power is read by the INA745BIRELR current sense resistor similar to the previous schematic. The output is controlled by a 7101SPDCQE toggle switch rated for 5A to handle the power for this branch. The two inputs located on the bottom right of the figure power the current sense resistor. The power comes from the same TPS62933DRLR 3.3V converter as in Figure 6. The two green connections are for the current sense resistor to communicate with the MCU.

Figure 7: 120VDC to 120VAC Branch

The 120VDC power line does not require voltage conversion since the power supply already operates at 120VDC. The line is protected by a 4230-T120-K0BE 25A circuit breaker. The breaker is rated at 25A to ensure the system does not trip while operating at 20A. The power is monitored by the INA745BIRELR current sense resistor. The output is controlled by a 32NT91-3 toggle switch which is rated for 20A. The schematic for this branch is shown in Figure 8. The power for the current sensor is supplied by the same 3.3V power source as the previous two schematics. The wires that deliver the power to the payload are 12 AWG to handle the 20A current.

Figure 8: 120VDC Branch.

# Chapter 3: Prototype and Testing

## 3.1 Prototype

Due to time limitations, the project's scope was changed. The new scope is delivering 28VDC to two outputs while reporting the voltage and current for each output. Each output is monitored by a Raspberry Pi Pico 2 mounted on a custom PCB with three current sensors. This PCB allows one Pico 2 to monitor 120VDC, 120VAC, and 28VDC. The ACS37800KMACTR-030B3-SPI hall effect current sensor was chosen to replace the INA745BIRELR sensors previously discussed. The new sensors have 0.85mΩ of internal resistance. This results in a similar power loss as the INA745 which has an internal resistance of 0.8mΩ. The INA745 is only able to measure current. The ACS37800 was chosen for the ability to monitor voltage and current simultaneously. With true voltage readings, power monitoring is more accurate. A 12V fan was added to the system to cool the 28V converter. The updated system diagram is shown in Figure 9.

Figure 9: Updated system diagram with two 28V payloads.

Figure 10 shows the assembled current sensor PCB. Each sensor level has two terminal blocks for connection. The lower terminal blocks are connected in series to the positive side of the load. The top terminal blocks are connected to the negative side of the load. The voltage at these terminal blocks is compared by the sensor to calculate the line voltage. Figure 11 is a schematic for connecting to the 28V sensor.

Figure 10: Current sensor PCB with Pico 2 attached using headers.

Figure 11: Schematic for connecting 28VDC sensor to a load for monitoring.

Each of the three current sensors on the PCB communicates with the attached Pico 2 over SPI. The information from each Pico 2 is then sent to the Raspberry Pi using I2C. Here, the user can view all available information at once. To set up a Pico 2 for use, code must be flashed using the micro-USB port and a computer. The code can be found in the team’s GitHub. In this code, there are variables for the voltage divider resistance values that each sensor uses. Each voltage level has a unique divider, so the user should make sure the appropriate resistance values are in the code to calculate the correct voltage. There is also a variable for wire resistance which can be adjusted to get higher accuracy. If the length or size of the wire between the sensor and the load is changed, a new resistance value should be found.

During initial testing, a challenge the team faced was calibrating the current sensors. The ACS37800 comes ready to measure AC voltage and current from the factory. To change this, the BYPASS_N_EN register was set to 1 and the number of samples for voltage and current averages was set to 1023. Gain and offset had to be calculated for the sensors as well. By setting gain and offset to zero and testing two points, they can be quickly calculated using a linear model.

A CFB600W-110S28 converter delivers the 28V output. This converter was chosen for its power requirements and pricing. This converter is a PCB mount, so a custom PCB was built seen on Fig. 12. The PCB uses terminal blocks located on the left and right for wire connections. An external input capacitor was added to reduce input ripple voltage and prevent the effect of input line inductance. Two external output capacitors were added to reduce output ripple and noise. A TVS diode was added at the input to protect the converter against surge or spike voltage and input reverse voltage. 4 additional capacitors were added connecting the 4 input and output terminals to the shield pattern to provide protection from noise. The converter is equipped with a heatsink mounted to the top of the converter to keep efficiency from dropping due to high temperatures.

Figure 12: 120VDC-28VDC converter on the custom PCB.

Figure 13: All components on the aluminum mounting plate.

## 3.2 Testing

Electrical, Software, and System Test Plans

Purpose

To ensure the created prototype is functional and adheres to all requirements discussed in earlier chapters, the prototype went through three categories of testing- Electrical, Software, and System Testing. There were four different Electrical Tests, two Software Tests, and two System Tests. The purpose of the Electrical Tests was to test the accuracy of the current sensing PCBs and converter performance at different wattages. The goal of the Software Tests was to ensure communication between Current Sensing PCBs and Raspberry Pi Pico 2 was established using the SPI protocol and communication between Pico 2 and Raspberry Pi was established using the I2C protocol. Moreover, the purpose of the System Tests was to verify that the prototype is within the size requirement, and the noise produced by the prototype is within range.

Scope

The scope of the tests include verification of different subsystems on all major electrical, software, and general system-level requirements. This involved checking if different electrical components such as the PCBs and Converters supplied voltage, current, and wattage to appropriate levels, if communication protocols were successful, and if the noise and size requirements were met. It is worth noting that certain limitations arose when testing the 28 VDC Converter capability of delivering 500W to load. That is because the testing environment did not have large enough DC Power Source needed for testing. This specific test cannot be completed until a bigger DC power source is located. Such limitation was anticipated; therefore, team members are arranging a testing session at the university’s high-power lab. As demonstrated in Table 3, the tests performed can be exclusively classified as First Article and Acceptance Tests. Additionally, only the Software Testing was verified via Demonstration, and the Size Test was verified via Inspection; others were verified via different types of Tests.

Table 3: Requirement/Verification Cross-Reference Matrix

Classes of Verification: I- First Article, II-Environmental, III-Acceptance Test, IV- None

Methods of Verification: A- Analysis, T-Test, D-Demonstration, I- Inspection

| Project Requirements | Test Names | Verification Class |  |  |  | Test Procedure Location |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV |  |
|  | Electrical Testing |  |  |  |  |  |
| Current Sensing Accuracy | Current Sensor – 28VDC, 20A | T |  | T |  | Appendix B 1.1 |
| 28VDC Output | 28VDC Converter - 500W | T |  | T |  | Appendix B 1.2 |
| MCU Power | 5VDC Output at 15W | T |  | T |  | Appendix B 1.3 |
| Fan Power | 12VDC output at 5.4W | T |  | T |  | Appendix B 1.4 |
|  | Software Testing |  |  |  |  |  |
| Current Sensor Code | SPI Code Test | D |  | D |  | Appendix B 2.1 |
| MCU Communication | I2C Code Test | D |  | D |  | Appendix B 2.2 |
|  | System Testing |  |  |  |  |  |
| Noise Level | Sound Test | T |  | T |  | Appendix B 3.1 |
| Size | Size Test | I |  | I |  | Appendix B 3.2 |

Test Explanation:

1.1 Current Sensor – 28VDC, 20A

This test is a First Article and Acceptance Electrical/Software Test. It was designed to check if the current sensing PCBs can read voltage and current being drawn by the load. To consider this test a pass, the PCB’s voltage and current reading accuracy needed to be within a 2% margin of error. As shown in Appendix B 1.1, the reading accuracy ranges between –1% to 1%. meaning this test passes. Although this test is not directly addressing any requirements, it is done to ensure the system is functional and can perform its assigned task.

1.2 28VDC Converter – 500W

This test is a First Article and Acceptance Electrical Test. This test is designed to address the prototype’s power requirements. One of the power requirements specified that the system is capable of delivering 500W at 28 VDC. In this test, a 28 VDC Converter was evaluated to see if it is capable of delivering 500W to the system load. A 28 VDC Converter is used to emulate the 28 VDC level. This test remains in progress because the DC power source used for testing cannot reach 600W. A solution to this limitation is to conduct this test in a high-power lab where a power source is capable of such bounds. Team members are arranging testing in the university’s high-power lab.

1.3 5VDC Output at 15W

This test is a First Article and Acceptance Electrical Test. This test is designed to address the power requirements of the Raspberry Pi 3B+ and the current sensor PCBs. In this test, a DFR0379 adjustable DC/DC converter was evaluated to see if it is capable of delivering 5V at 15W to the Raspberry Pi and PCBs. As shown in Appendix B 1.3, the converter was capable of delivering 15.175W, meaning this test passed. Although this test does not address any specific requirements, it is done to ensure the Raspberry Pi and current sensing PCBs get required power.

1.4 12VDC output at 5.4W

This test is a First Article and Acceptance Electrical Test. This test is designed to address the power requirements of the cooling fan. In this test, a DFR0379 adjustable DC/DC converter was evaluated to see if it is capable of delivering 12V at 5.4W to the cooling fan. As shown in Appendix B 1.4, the converter was capable of delivering 5.419W, meaning this test passed. Although this test does not address any specific requirements, it is done to ensure the system does not overheat during regular usage.

2.1 SPI Code Test

This is a First Article and Acceptance Software Test, and it is verified via Demonstration. The goal of this Demonstration was to prove the SPI communication between Raspberry Pi Pico 2 and the current sensing PCB can be established. The terminal output displaying the raw and calculated current and voltages indicate that the SPI communication was established between the two components. Although this test does not address any specific requirements, it is conducted to verify different subsystems can communicate clearly.

2.2 I2C Code Test

This is a First Article and Acceptance Software Test, also verified via Demonstration. The goal of this Demonstration was to prove the I2C communication between Raspberry Pi and Pico 2 can be established. This test is considered a failure as communication between the two components could not be established. The testing is in progress; the Pi master code and Pico slave code are being manipulated as well as the hardware connections to establish communication between the two components. Similar to the other Software Test, this test does not address any specific requirements; it is conducted to verify different subsystems in the prototype can communicate clearly.

3.1 Sound Test

This is a First Article and Acceptance System Test; it is verified via curated testing. The goal of this test is to ensure the system meets the noise level requirements defined in the earlier stage of this project. The designed prototype must produce minimal noise; the acceptable range is 25-64 dB at 1 meter. Meaning the produced noise must be something between a whisper and a distant conversation. Conducting this test involved setting up the entire system along with the fan and measuring the sound from 1 meter away; the decibel level was measured using a noise monitoring application. This test is considered a success because at 1 meter away, the noise level record is 64 dB; the noise level is within the defined range.

3.2 Size Test

This is a First Article and Acceptance System Test; it is verified via Inspection. This test addresses the size requirements of the project. The goal is to ensure all components must be contained within the mounting plate, and the plate size does not exceed 535 x 600 x 195 mm (or 21 x 23.62 x 7.68 in) dimensions. Upon assembly, the prototype size is 457.2 x 457.2 x 161.93 mm (or 18 x 18 x 6.375 in). This demonstrates that the prototype measurements are within range, meaning the test passes.

# Chapter 4: Final Design

The final prototype can be found in Figure 14. The components are mounted onto an 18”x18” aluminum 6061 sheet that is 0.063” thick. The side panel of the same material is used to hold the toggle switch and the circuit breaker is 6" x 12". The prototype uses 12-gauge ring connectors to connect the system to power and the load. The 28VDC converter, current sensor PCB, the two 20W DC/DC adjustable converters, and the Raspberry Pi 3B+ are all mounted to the aluminum sheet using M3 20mm hex head screws with aluminum split lock washers and medium-strength steel nylon-insert locknuts used as fasteners. 10mm Nylon PCB standoffs were placed between the components and mounting sheet to prevent short circuits and improve air flow to the bottom of the converters. The wires used to connect the 28VDC converter to the current sensor PCB, circuit breaker, toggle switch, the payload, and the DC power source were 12-gauge wires to handle the required current for the system. 20 AWG wire is used to connect the 28V converter to the DC/DC adjustable converters due to lower power draw.

Figure 14: Final Prototype

The converters were cooled using a 12V, 5.4W Sunon fan. The placement of the fan allows airflow through the top and bottom of each converter as well as through the 28VDC converter heat sink. The fan is powered by a DFR0379 adjustable DC/DC converter set to 12V. The second DC/DC converter is unused in the current configuration. This converter is set to 5V and is intended to power the Raspberry Pi 3B+ and the Pico 2. Figure 15 is a simplified block diagram labeling each component present in the prototype as well as showing basic connections.

Figure 15: Final Prototype Block Diagram

Figure 16 is a block diagram demonstrating proper connection of a DC supply to a current sensor and load for power monitoring. Power goes through the sensor on the positive side as well as another connection on the negative side. These two points are what is compared to measure the line voltage. The power supply is symbolic of the appropriate source of power for the given load. This diagram shows the connection to the 28V sensor on the PCB. For 28V, the 120-28VDC converter would be the power supply. For 120VAC, the AC inverter would be used. Both the positive and negative side connection can be connected in either port of the terminal block. However, reversing the positive side will cause the current reading to be negative. To solve this, swap the connection order or change the Raspberry Pi’s code to define current direction in the sensor.

Figure 16: Sensor PCB connection

This prototype performs well in reading any currents in the expectd range. Voltage readings have been shown to have a linear error depending on the current being drawn. Further calibration and testing can eliminate this error. A linear model was used in calibration of the current and the voltage readings. A gain and offset can be found using equations 1 and 2.

Gain = ActualHigh-ActualLowMeasuredHigh-MeasuredLow	  	            (1)

Offset = ActualHigh - MeasuredHigh⋅Gain		(2)

In these equations, “Actual” refers to the real voltage and current seen by the load. “Measured” is the output reading from the sensor. The resistance of the wires between the sensor PCB and the load must also be accounted for. In the software, there is a variable value that must be set whenever wire length is changed. The full process for calibration can be found in the next chapter.

Figures 17, 18, and 19 display the layout and connections of the sensor PCB. This PCB is designed to accommodate 120VAC, 120VDC, and 28VDC outputs. More sensors could be added to the design as needed in the future. The only limitations of expansion would be size and having open pins on the Pico 2 for Chip Select (CS) connections to each sensor. Currently, there are enough extra pins for 18 more sensors to connect to the Pico 2. In Figure 19, the voltage divider for 28V is shown. Each sensor has a divider designed specifically for its voltage. Using a higher voltage on the 28V sensor will result in incorrect voltage readings. Details on sizing this divider can be found in the next chapter as well as the ACS37800 datasheet. The JST connectors and terminal blocks are all labeled on the PCB for easy use. The three connectors on the left in Figure 17 as well as the through holes next to the headers are all extra connections. These can be used during testing to identify any issues. The through holes also serve as a physical way to change the pin connections to the Pico 2 if needed.

Figure 17: Sensor PCB design

Figure 18: Sensor PCB version 2. Modified from the original to use a Pico 2 as well as using JST connectors for more convenient use.

Figure 19: 28VDC sensor connections for PCB

Figures 20 and 21 show the experimental percent error while testing the sensor in the lab. Voltage and current errors were projected out to the maximum current (18A) and found to be acceptable. The current sensor showed a relatively high error at currents at or less than 0.5A. This quickly improved to less than 1% error at 1A. The data for these tests can be found in Appendix C1.

Figure 20: 28VDC sensor voltage measurement accuracy.

Figure 21: 28VDC sensor current measurement accuracy.

Figure 22. Includes the PCB design used to create the 28VDC converter. Figure 23. Reflects the PCB design and shows the pin connections of the converter. The converter uses capacitors located across the input and output terminals of the converter to reduce the DC ripple and create a steady DC power line. A transient voltage suppressor diode is attached across the input terminal to protect the unit against surge or spike voltage and input reverse voltage. Capacitors C4, C5, C6, and C7 connect the four terminals of the converter to the shield pattern located under the converter. The shield pattern protects the converter from electromagnetic noise. When testing the converter with the system, the efficiency of the converter reached 80% efficiency. When the converter was tested by itself, the converter reached nearly 85% efficiency.

Figure 22: 28VDC Converter PCB Design.

Figure 23: 28VDC Converter PCB Design.

The prototype consists of a two-step communication style. Between the Raspberry Pico 2 and the current sensing PCBs, the communication follows the Serial Peripheral Interface (SPI) protocol while the Raspberry Pi and the Pico 2 communicate using the Inter-Integrated Circuit (I2C) protocol. The Pico plays a dual role – Master in the SPI protocol and Slave in the I2C protocol. The choice of protocols leverages the specific performance advantages of each interface. For instance, SPI ensures that data can be sent and received simultaneously. This is an important quality for our design because the prototype must be able to output one set of current and voltage while reading another set within an interval of 0.5 seconds; efficiency is of utmost importance. In fact, this is one of the main reasons for choosing SPI-based ACS37800 current sensors.

On the other hand, the I2C protocol is a better choice for the Pico to the Pi communication as it allows for minimal pin usage and having built-in addressing. The I2C allows scaling up to 127 devices using only two buses. This will be beneficial when the prototype incorporates additional voltage levels.

To understand the implementation of the two protocols, the algorithms in “Full_Code(I2C+SPI)” and “Pico Master Code” must be explored. “Full_Code(I2C+SPI)”- configure pins for SPI and I2C, define sensor calibration constants. The code’s SPI portion- the raw values are read into 0x20 to build a 32-bit response (the first 16 is current and the latter in voltage) then the sensor is deselected. The code’s I2C portion- Pico stores Pi request for voltage and current, Pi requests data, Pico checks said request, then sends the data stored. “Pico Master Code” - Pi creates specific requests; Pi reads the given data and converts given value.

In order to establish communication, it is imperative that the Pi and the Pico are appropriately powered. Then, using an Integrated Development Environment (IDE) such as Visual Studios Code (VS Code), the Pico must be flashed with the code presented in “Full_Code(I2C+SPI)”. Additionally, the Raspberry Pi must be configured to enable I2C communication, then install the smbus2 library, and a python file must be created to include the code in “Pico Master Code”. This establishes the two-step communication.

This prototype has several areas that are important for future improvement. Firstly, the 5V DC converter should be integrated to power the Raspberry Pi and Pico 2. The system currently only operates while a laptop is connected to provide power. Our team had issues using the 5V DC converter for power. The team’s theory was that ground was floating and causing voltage readings to max out at 42V. Further testing should be done to determine the exact cause of high voltage readings. Current readings function correctly. The team suggests using a diode between the 5V converter and the Raspberry Pi to prevent backfeeding. The next improvement would be to add an external display. With its own power and display, the Raspberry Pi could constantly output measurements to the display which would eliminate the need to connect to a laptop. The PCB is ready for adding 120VDC and 120VAC outputs, so modifying the process currently in the software and flashing the Pico 2 and Raspberry Pi would be all that is needed to add those outputs.

In addition to the physical aspect of the prototype, improvements to the code were necessary to facilitate I2C communication. Previously, the communication between the Pico and the Pi were not synchronized correctly so the two components could not connect; leading to the failure in Test 2.2. When the Pi sent specific requests, the Pico would acknowledge said request partially and not fully. This led to no communication between the Pi and the Pico. Upon changing how the Pico handles Pi’s requests, communication could be established. Beyond this change, an area of future improvement would be to modify the code in “Full_Code(I2C+SPI)” and “Pico Master Code” to accommodate the reading of multiple current sensors simultaneously.

The cost of the prototype can be found in Table 3. A majority of the cost come from the custom PCBs used for the 28V converter and current sensors. A minimum of 5 PCBs can be ordered for each board, hence why there are 5 28V converter PCBs. The components of the 28VDC converter PCB were hand soldered using the components ordered in the orange section of the table. These components were ordered from mouser. The current sensor was pre-soldered by JLCPCB when it was ordered. The pre-assembled boards cost more than the others seen in the green section of the table. Two of these boards were ordered in case of damage to one of them. The components in the gray section of the table were ordered from digikey and contained the electrical components and microcontrollers. The items in the blue were ordered from McMaster Carr and contain the mounting and assembling components. The green section contains the PCBs ordered from JLCPCB.

Table 3: Prototype Cost

| Item Description | Manufacturer Part No. | Unit of measure | Price (individual) | Quantity | Price (total) | Shiping Cost |
| --- | --- | --- | --- | --- | --- | --- |
| Raspberry Pi | SC0073 | Each | $40.00 | 1 | $40.00 |  |
| Circuit Breaker 28 VDC | 1019986 | Each | $17.26 | 1 | $17.26 | $- |
| Raspberry Pi Pico 2 | SC1631 | Each | $6.12 | 1 | $6.12 |  |
| 28VDC to 5VDC and 3.3VDC | DFR0379 | Each | $4.90 | 2 | $9.80 |  |
| Toggle Switch 28 VDC | S333 | Each | $14.84 | 1 | $14.84 | $- |
| Converter Heat Sink | 2034-3501-ND | Each | $9.93 | 1 | $9.93 |  |
| 120VDC to 28VDC Converter | CFB600W-110S28 | Each | $249.41 | 1 | $249.41 |  |
| Cooling Fan | EEC0251B1-000U-A99 | Each | $16.29 | 1 | $16.29 |  |
| PCB Standoffs | 25510 | Each | $0.58 | 20 | $11.60 |  |
|  |  |  |  |  |  | 7.97 |
|  |  |  |  |  | Total | $383.22 |
| 220uF Capacitor | 860241081009 | Each | $5.77 | 1 | $5.77 |  |
| 10uF Capacitor | T55D106M050C0090 | Each | $2.49 | 1 | $2.49 |  |
| 1uF Capacitor | CL21B105KBFNNNE | Each | $0.11 | 1 | $0.11 | $- |
| 10nF Capacitor | C323C103K2R5TA | Each | $0.41 | 2 | $0.82 | $- |
| 4.7nF Capacitor | CK45-B3DD472KYVNA | Each | $0.71 | 2 | $1.42 | $- |
| Diode | SMF4L160A | Each | $0.58 | 1 | $0.58 |  |
| Terminal | 1711026 | Each | $1.76 | 2 | $3.52 |  |
|  |  |  |  |  |  | 11.42 |
|  |  |  |  |  | Total | $26.13 |
| M3 Screws | 98511A734 | Package | $14.78 | 2 | $29.56 |  |
| Lock Nuts | 90576A811 | Package | $10.70 | 1 | $10.70 | $- |
| Lock Washers | 91013A711 | Package | $5.36 | 1 | $5.36 | $- |
| Aluminum Mounting Sheet | 89015K189 | Each | $34.29 | 1 | $34.29 | $- |
| DIN rail | 8961K86 | Each | $4.47 | 1 | $4.47 |  |
| Aluminum Side piece | 89015K184 | Each | $6.25 | 1 | $6.25 |  |
| Brackets | 1556A39 | Each | $2.87 | 2 | $5.74 |  |
|  |  |  |  |  |  | 18.58 |
|  |  |  |  |  | Total | $114.95 |
| 28V converter PCB |  | Package | $5.28 | 5 | $26.40 |  |
| Current Sensor PCB |  | Each | $5.14 | 3 | $15.42 |  |
| Current Sensor PCB Assembled |  | Package | $63.82 | 2 | $127.64 |  |
|  |  |  |  | Subtotal | $169.46 |  |
|  |  |  |  | Shipping | 29.56 |  |
|  |  |  |  | Import Taxes | $59.31 |  |
|  |  |  |  | Total | $258.33 |  |
| Prototype Grand Total | $782.63 |  |  |  |  |  |

Table 3. shows the cost of the prototype if the 120VDC and 120VAC were incorporated. The biggest cause in the price jump comes from the 120VAC converter. Other additional costs include a 120VDC circuit breaker, 120VDC toggle switch, 120VAC toggle switch, and 120VAC circuit breaker.

Table 4: Prototype Three Voltage Levels Cost

| Item Description | Manufacturer Part No. | Unit of measure | Price (individual) | Quantity | Price (total) | Shiping Cost |
| --- | --- | --- | --- | --- | --- | --- |
| Raspberry Pi | SC0073 | Each | $40.00 | 1 | $40.00 |  |
| Circuit Breaker 28 VDC | 1019986 | Each | $17.26 | 1 | $17.26 | $- |
| Raspberry Pi Pico 2 | SC1631 | Each | $6.12 | 1 | $6.12 |  |
| 28VDC to 5VDC and 3.3VDC | DFR0379 | Each | $4.90 | 2 | $9.80 |  |
| Toggle Switch 28 VDC | S333 | Each | $14.84 | 2 | $29.68 | $- |
| Converter Heat Sink | 2034-3501-ND | Each | $9.93 | 1 | $9.93 |  |
| 120VDC to 28VDC Converter | CFB600W-110S28 | Each | $249.41 | 1 | $249.41 |  |
| Cooling Fan | EEC0251B1-000U-A99 | Each | $16.29 | 1 | $16.29 |  |
| PCB Standoffs | 25510 | Each | $0.58 | 20 | $11.60 |  |
| 120VAC Circuit Breaker | 1019914 | Each | $17.26 | 1 | $17.26 |  |
|  |  |  |  |  |  | 7.97 |
|  |  |  |  |  | Total | $415.32 |
| 220uF Capacitor | 860241081009 | Each | $5.77 | 1 | $5.77 |  |
| 10uF Capacitor | T55D106M050C0090 | Each | $2.49 | 1 | $2.49 |  |
| 1uF Capacitor | CL21B105KBFNNNE | Each | $0.12 | 1 | $0.12 | $- |
| 10nF Capacitor | C323C103K2R5TA | Each | $0.41 | 2 | $0.82 | $- |
| 4.7nF Capacitor | CK45-B3DD472KYVNA | Each | $0.71 | 2 | $1.42 | $- |
| Diode | SMF4L160A | Each | $0.58 | 1 | $0.58 |  |
| Terminal | 1711026 | Each | $1.76 | 2 | $3.52 |  |
| 120VDC Circuit Breaker | CX1-B0-14-625-52B-10-C | Each | 58.31 | 1 | $58.31 |  |
| 120VDC Toggle Switch | 32NT91-3 | Each | $45.74 | 1 | $45.74 |  |
|  |  |  |  |  |  | 9.39 |
|  |  |  |  |  | Total | $131.09 |
| 120VDC to 120VAC Inverter | XP 6-1-I-6-1 | Each | $693.99 | 1 | $693.99 |  |
| M3 Screws | 98511A734 | Package | $14.78 | 2 | $29.56 |  |
| Lock Nuts | 90576A811 | Package | $10.70 | 1 | $10.70 | $- |
| Lock Washers | 91013A711 | Package | $5.36 | 1 | $5.36 | $- |
| Aluminum Mounting Sheet | 89015K189 | Each | $34.29 | 1 | $34.29 | $- |
| DIN rail | 8961K86 | Each | $4.47 | 1 | $4.47 |  |
| Aluminum Side piece | 89015K184 | Each | $6.25 | 1 | $6.25 |  |
| Brackets | 1556A39 | Each | $2.87 | 2 | $5.74 |  |
|  |  |  |  |  |  | 18.58 |
|  |  |  |  |  | Total | 114.95 |
| 28V converter PCB |  | Package | $5.28 | 5 | $26.40 |  |
| Current Sensor PCB |  | Each | $5.14 | 3 | $15.42 |  |
| Current Sensor PCB Assembled |  | Package | $63.82 | 2 | $127.64 |  |
|  |  |  |  |  | Subtotal | $169.46 |
|  |  |  |  |  | Shipping | 29.56 |
|  |  |  |  |  | Import Taxes | $59.31 |
| Grand Total | $1,613.68 |  |  |  | Total | $258.33 |

Table 5 shows the cost of added payloads. Table 4 shows the cost of the system for one payload. Since the Voltage levels are already setup, there is no need for additional converters. One toggle switch and breaker are needed per voltage level per payload. Each additional payload will also need a current sensor PCB and Pico 2 to read the power draw at that payload. When ordering the current sensor PCB, there is a minimum amount of 5 PCBs per board. The price on the table reflects the cost of one board with the shipping and import tax being split five ways.

Table 5: Cost Per Payload

| Item Description | Manufacturer Part No. | Unit of measure | Price (individual) | Quantity | Price (total) | Shiping Cost |
| --- | --- | --- | --- | --- | --- | --- |
| Circuit Breaker 28 VDC | 1019986 | Each | $17.26 | 1 | $17.26 | $- |
| 120VAC Circuit Breaker | 1019914 | Each | $17.26 | 1 | $17.26 |  |
| Raspberry Pi Pico 2 | SC1631 | Each | $6.12 | 1 | $6.12 |  |
| Toggle Switch 28 VDC | S333 | Each | $14.84 | 2 | $29.68 | $- |
|  |  |  |  |  |  | 6.99 |
|  |  |  |  |  | Total | $77.31 |
| 120VDC Circuit Breaker | CX1-B0-14-625-52B-10-C | Each | 58.31 | 1 | $58.31 |  |
| 120VDC Toggle Switch | 32NT91-3 | Each | $45.74 | 1 | $45.74 |  |
|  |  |  |  |  |  | 9.39 |
|  |  |  |  |  | Total | $113.44 |
| Current Sensor PCB Assembled |  | Package | $63.82 | 1 | $63.82 |  |
|  |  |  |  |  | Subtotal | $63.82 |
|  |  |  |  |  | Shipping | 5.91 |
|  |  |  |  |  | Import Taxes | $22.34 |
| Total | $169.38 |  |  |  | Total | $92.07 |

# Chapter 5: User Manual

Product description and goal

The Starlab power delivery system is designed to provide quick access to many voltages to power a wide variety of payloads. The current prototype has one 28V output, but future iterations will provide several voltages for each payload. To use the prototype, physical wire connections must be made as well as flashing code to the Raspberry Pi and Pico 2.

Product Usage

Requirements

Software Dependencies/Requirements:

Python3, smbus2 library, IDE such as VS Code, Raspberry Pi OS.

Hardware Dependencies/Requirements:

Micro-USB to flash code.

Safety Requirements

The 28V converter for is system was safely tested up to 350W. The entire system was tested up to 200W. The system should be carefully tested up to its maximum output of 500W before real-world use.

To use the system, ensure the toggle switch is in the off position before connecting or disconnecting the power source or payload. In the case of the circuit breaker tripping, turn off the power output to the payload before flipping the breaker back to the on position. Correct the issue that caused the overcurrent before attempting to turn the system back on. Always ensure the power supply is off before changing any connections. All connections should be secure and not contacting any conductive surfaces before power is turned on.

Never connect high voltage to the PCB except at the screw terminal blocks. Do not power the Pico 2 or Raspberry Pi with anymore than 5.15V.

How to operate the Starlab Power Delivery System

The system is powered using the wires connected to the 120VDC side of the 28V converter. This side is located nearest to the fan. The polarity of each input wire can be found on the side of the green screw terminal blocks. Before connecting the system to the power source, ensure the toggle switch located on the front panel is in the off position with the level sticking straight out. Check that the circuit breaker that is mounted near the switch is turned on. Connect the DC source with the corresponding polarity wire. Connect the positive terminal of the payload to the wire connected to the toggle switch. Connect the negative terminal of the payload to the wire connected to the current sensor PCB. To power the payload, flip the toggle switch down. The Raspberry Pi and Pico 2 are flashed with the code written for 28V output. If future changes are made, a laptop should be connected to each controller individually using micro-USB and flashed. Once they are flashed, they will retain the information even when powered off. Reflashing is only necessary when changes are made to the code.

Basic trouble shooting and common issues

Problem 1: No Power to the Payload

Check all connections are secure and as previously described. Wires should not be touching any conductive surfaces or each other. The circuit breaker must be in the “on” position and the switch should be down to be in the “on” position.

If a clicking sound is heard from the 28V converter, this indicates insufficient power being provided. Check source voltage.

Problem 2: No Voltage or Power Readings

Check that both the Raspberry Pi and Pico 2 have 5V power. Check that the three wires for I2C communication between the Pico 2 and the Raspberry Pi are securely connected. Try reflashing the code on each controller if the problem persists.

Problem 3: Incorrect Voltage or Power Readings

Check that the most up-to-date code is flashed on both controllers. Check that all wire connections are secure and not touching any conductive surfaces. Ensure that the sensor that is being used is designed for that voltage or a voltage slightly higher than what you are measuring. A multimeter can be used to check for any bad resistors that form the voltage divider.

In the code, check the wire resistance, gain, and offset values. If necessary, go through recalibration using an electronic load. The recalibration procedure is in the Enineering Troubleshooting section.

Engineering Troubleshooting report

Problem 1: No Power to the Payload

Faulty power source connection:

Fault could either come from the source side or the terminal block. Ensure proper connection at both ends. If problem is at the screw terminal end. Use a Philips screwdriver to tighten, loosen, or adjust the input wire until proper connection is achieved.

Fault between wire connections of subcomponents:

Wires could be loose or disconnected between the different components of the system. Test the wire connections between the positive output terminal of the 28VDC converter and current sensor PCB, the current sensor PCB and the circuit breaker, the circuit breaker and the switch, the switch and the positive terminal of the payload, the negative terminal of the payload and the current sensor PCB, and the current sensor PCB to the negative output terminal of the 28VDC converter. If any of these connections prove faulty, adjust the wire connections on both sides using the screw block terminals and a Philips screwdriver.

Circuit Breaker Trip:

If the payload tries to draw too much power from the system, the circuit breaker will trip to protect the components in the system. Ensure the toggle switch is in the off position before resetting the breaker.

Undervoltage Protection:

The converter in this system comes with an under-voltage lockout (UVLO) feature. If the source voltage drops below 43VDC, the converter will shut down. This is associated with a clicking noise coming from the converter. Ensure the source voltage is at 120VDC before using again.

System Overheated:

The converter has a built-in temperature protection circuit to protect against thermal damage. The converter will shut down when the reference temperature is exceeded. Shut off the system until the converter cools below the reference temperature. The converter will restart once the temperature drops.

Problem 2: No Voltage or Power Readings

First, follow the steps outlined in the basic troubleshooting section. The user can directly connect to the Pico 2 to determine if the issue is with the I2C connection or the Raspberry Pi. If the Pico 2 reads correctly, check the I2C connection and Raspberry Pi code. If the Pico 2 is not reading correctly, the physical connection is not complete or a mistake in the code was made. A multimeter can be used to check the “3.3V_OUT” pin on the PCB. If this reads below ~3V and/or the Pico 2 is extremely hot, immediately turn off the power. There is an improper connection that can burn out the Pico 2 if left on long enough. If the Pico 2 will not output 3.3V, it has been burnt out and needs to be replaced.

If trying to use the 5V DC converter to power the Raspberry Pi and Pico 2, ensure a diode is placed between the converter and the controllers to prevent backfeeding. If current readings are correct but voltage is not, check that all devices have a proper common ground for the voltage divider on the PCB to work properly. If these fail, attempt recalibration.

Problem 3: Incorrect Voltage or Power Readings

First, follow the steps outlined in the basic troubleshooting section. If monitoring AC voltage, the register “BYPASS_N_EN” must be changed. See ACS37800 datasheet for details. In the file “Full_Code(I2C+SPI), this is done on lines 133-142. If recalibration is needed, perform the following:

- Connect to the Pico 2 using micro-USB.

- Flash the latest code using Visual Studio Code

- Connect the system to a DC electronic load and a DC power supply.

- Set the gain and offset values in the Pico 2 code to 0.0.

- Measure the wire resistance between the load and sensor PCB on both sides. The sum of these is the total resistance. Assign this value to the variable “Wire_resistance”.

- Set the power supply to a low voltage (a few volts) with no current. Note the voltage that the load sees as well as the measured voltage from the sensor. These will likely not be the same.

- Set the power supply to the maximum voltage that will be measured. Again, note the actual and measured values.

- Using equation (1) in chapter 4, find a gain value. In the code, set “SensV_Gain” to this value.

- Using equation (2) in chapter 4, find the offset. In the code, set “SensV_Offset” to this value. This concludes voltage calibration

- Set the voltage to any value and do not change. Repeat steps 5-8 while now adjusting the current drawn by the load. This will find gain and offset for the current. The variables to change are named “SensI_Gain” and “SensI_Offset”.

- Test the calibration by setting the voltage to a chosen value with zero current. Increase the current in steps and note the small voltage drop seen at the load. If, at any amount of current, the voltage measured deviates more than desired, the variable for wire resistance needs to be adjusted. When operating at high current, if the measured voltage is high, resistance should be increased. If the measured voltage is low, resistance should be decreased. Adjust the resistance value until an acceptable percent error is achieved.

Problem 4: Issues Connecting Pico 2 and Raspberry Pi

If issues arise when connecting Pico 2 to the Raspberry Pi, check to see if the Raspberry Pi pins defined in code are being used correctly.

- Verify I2C is enabled in the Pi.

- Ensure that the correct I2C addresses are being used by both parties. If Pi uses register 0x42, Pico must as well.

- Use the command i2cdetect -y 1 to check if the connection has been established. Do not use Bus 0 as it is not connected to any GPIO.

- Verify the transaction order is correct. Pi must write request type then read the response. Error in order will produce in correct data.

# Chapter 6: Project Management, Summary, and Conclusion

## 6.1 Project Management

The team utilized various methods of management and communication throughout the course of this project. One of the notable tools was using project management software such as Jira to create and keep track of tasks and subtasks. This was helpful in dividing responsibilities and work toward larger milestones. Appendix D contains the Gantt Chart which shows the progression of this project. Beyond that, the team held weekly hour-long virtual meetings during the first semester to touch base on each other's progress. This was extremely beneficial given that a major part of Capstone I was conducting research and documenting them. In the latter half of the project, the team regularly met at the Capstone Lab to research/work on the project’s subsystems and stayed in contact with the OSU Starlab Team every few weeks. Over the course of both semesters, communication via text message was consistent among team members.

In terms of potential problems, the team encountered a delay in receiving detailed information from sponsors during Capstone I. This hindered the team’s progress a certain amount by the end of Semester 1, but recovery was made swiftly through extra time spent on the project. However, a major challenge the team faced during Capstone II was implementing the communication protocol. Between finding the appropriate component, protocol changes, and code issues, the team’s progress was impacted a significant amount; project scope needed to be reduced. Debugging the code and detailed research helped as a team helped in mitigating this issue.

In terms of suggestions for the future, it is imperative to do detailed research on how the embedded system works as a whole and separately. Another suggestion is to learn PCB design very well.

## 6.2 Summary

Over the course of Capstone I and II, Team 16 has worked with Airbus and OSU Starlab Team to create a power system that can supply voltage levels of 120 VAC, 120 VDC, and 28 VDC to various channels. Due to time constraints, the project scope was scaled down to handle a voltage level of 28 VDC. The prototype is able to undertake this task at its current stage. Furthermore, the system’s current and voltage levels can be monitored using the established SPI and I2C communication protocols. The voltage and current are measured every 0.5 seconds to ensure their accuracy. Since the change in project scope took place later on, the current sensor PCB is designed to accommodate 120 VDC/VAC and 28 VDC. The current prototype consists of different voltage converters, fan for cooling purposes, current sensor PCB with the Pico, Raspberry Pi, Circuit Breaker, and switch to control the device. The current prototype has designated space for 120 VAC inverter which will be necessary for the 120 VAC voltage level. The Raspberry Pi needs to be powered using a laptop in order to function properly.

If the project is further explored in the future, the main goal would be to incorporate the voltage level 120 VDC and 120 VAC. Some modifications to the communication protocols can be implemented along with certain layout changes in the current prototype can make it possible.

## 6.3 Conclusion

[Write a conclusion stating any future work or recommendations for others to know about your final solution. In addition, each individual must write a paragraph explaining their main contribution to the project, lesson learned, and how they overcame and improved their weaknesses through this project. Think of it as a reflection on the past year.]

Future work for this project would include expanding the power system to incorporate the 120VDC and 120VAC power levels. The current system is designed for a single payload, so additional expansion would be needed to support multiple payloads in the future. There would also have to be alterations in the code for the raspberry pi to read from multiple current sensor PCBs.

Greyson Donley: My main contributions to the project were the sensor PCB design as well as the initial SPI code between the sensors and the Pico 2. I learned a lot about PCB design. I went from not knowing the first thing about it to creating and implementing a successful design. The software side of the project was the largest challenge. Microcontrollers are a very different world of coding, and I did not have much experience. With many component and design changes throughout the year, it felt like the team was always trying to learn how to use new parts. These changes set us behind schedule at times. Even things like compiling code to write to the ESP32’s ending up soaking up much of our time and we ended up pivoting to the Pico 2. I believe my leadership and communication skills have improved as I lead the team over the course of this project. I had to become comfortable with being the main communicator with my team and with sponsors. I also feel that I am better at anticipating issues that will arise from the experiences we have had in capstone. Many problems and delays happened. We had to learn how to adapt and keep moving forward through the tough stages.

Lucas Brickner: My main contributions were the 28V converter and the assembly of the final prototype. A major lesson I learned during this project was the amount of thought that goes into designing a system that can handle higher levels of power. I became very aware of how much voltage and current each part of the system had to handle and how to pick parts based on these safety ratings on datasheets to ensure there was no system failure during testing and standard use. This was also my first time developing a PCB, so this was a skill I had to learn during the process. My communication skills greatly improved during this project. Working with sponsors and teammates showed me how important it is to give updates so people can stay in the loop. It is also important to ask teammates for clarification on the aspects of the project they are working on so you can incorporate your side. Overall, this project has taught me how to be a professional and how to work through setbacks when they appear in a project.

Fatima Sayeed: My contribution to this project is the I2C communication between the Pico and the Raspberry Pi. My responsibilities included both I2C and SPI communication; however, I was not successful in delivering the latter. There are a lot of new things I learned from being part of this project, but a major one being how to work with microcontrollers and the realm of embedded systems. Prior to this project, I have had minimal experience working with embedded systems. This being part of this project allowed me to learn and explore the usage of microcontrollers in greater detail. Part of this process has been overwhelming in terms of selecting the appropriate components, but it has exposed me to lots of different types of communication and technology. I have learned how to do detailed research for embedded systems in great detail. Although I cannot say I have done everything perfectly, I have been able to learn a lot throughout this process. Additionally, I have improved my skills in technical writing over the course of this project and Capstone. This project has been a massive learning opportunity for me overall.

Appendix A: Microcontroller Selection Decision Matrix

|  |  | Raspberry Pi 3 Model B+ |  | ARDUINO MEGA2560 |  | Teensy 4.1 |  | STM32 NUCLEO-F401RE |  | ESP32 WITH HEADERS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Criteria | Weight (0-1) | Score | Weighted Score | Score | Weighted Score | Score | Weighted Score | Score | Weighted Score | Score | Weighted Score |
| Cost | 0.4 | 4 | 1.6 | 5 | 2 | 3 | 1.2 | 1 | 0.4 | 2 | 0.8 |
| I2C | 0.7 | 5 | 3.5 | 1 | 0.7 | 4 | 2.8 | 3 | 2.1 | 2 | 1.4 |
| CPU Speed | 0.5 | 5 | 2.5 | 1 | 0.5 | 4 | 2 | 2 | 1 | 3 | 1.5 |
| Memory | 0.6 | 5 | 3 | 1 | 0.6 | 3 | 1.8 | 2 | 1.2 | 4 | 2.4 |
| I/O Types | 0.8 | 4 | 3.2 | 3 | 2.4 | 5 | 4 | 1 | 0.8 | 2 | 1.6 |
| Total | 3 | 23 | 13.8 | 11 | 6.2 | 19 | 11.8 | 9 | 5.5 | 13 | 7.7 |

Appendix B1: Electrical Test Plans

| B 1.1 Current Sensor – 28VDC, 20A |  |  |
| --- | --- | --- |
| Method of Verification: Test |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 4/2/26 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: Raspberry Pi Pico 2, Computer, Current Sensor PCB, DC Power Supply, DC Electronic Load |  |  |
| Test Engineer: Greyson Donley Witness: Lucas Brickner |  |  |
| Criteria for Success: ☒ Passes – Sensors read voltage and current being drawn by load within 2% ☐ Fails – Sensors unable to read voltage and current being drawn by load within 2% |  |  |
| Test Procedure: - Retrieve code located in Starlab GitHub labeled “PICO_SPI”. - Connect Pico to headers on the sensor PCB. - Connect Pico to computer via micro-usb. - Flash the "PICO_SPI” file to the Pico using Visual Studio Code. - Connect DC power supply and load to the 28V sensor following the schematic “28VDC Sensor Connection”. - Set DC load to the desired current for reading. - Set DC power supply to the desired voltage for reading. Ensure the power supply current is set at or above the load’s current setting. - Turn on the DC power supply. - Turn on the DC load. - Record voltage and current readings from the terminal on the laptop to verify accuracy. |  |  |
| Test Results: (Left: load, right: sensor reading) Percent Error: 2.98V, 1A Voltage: 0%, Current: 0% 2.85V, 8A Voltage: 0%, Current: 0% 14.83V, 8A Voltage: 0.07%, Current: 0% 27.81V, 8A Voltage: 0.04%, Current: 0% 7.80V, 10A Voltage: 0.13%, Current: 0.1% 7.56, 20A Voltage: -0.26%, Current: -0.1% |  |  |
| Action Items: |  |  |
| Test Engineer: GWD, 04/02/26 |  | Witness: LGB, 04/02/26 |

| B 1.2 28VDC Converter - 500W |  |  |
| --- | --- | --- |
| Method of Verification: Test |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 4/2/26 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: Example: 120VDC Power Supply, Multimeter, Electronic Load |  |  |
| Test Engineer: Lucas Brickner Witness: Greyson Donley |  |  |
| Criteria for Success: ☐ Passes – Converter delivers 500W to the load ☐ Fails – Converter unable to deliver 500W to the load |  |  |
| Test Procedure: - Locate schematic “28VDC Converter Test Setup” - Connect the power supply to the input of the buck converter. - Connect the output of the converter to the electronic load. - Set electronic load to draw 500W (17.86A) - Set the power supply to 120VDC and 4.5A - Turn on power supply - Turn on the electronic load - Record amperage to the load to verify wattage requirement |  |  |
| Test Results: 1A: Power Source: Load: Efficiency Calculations: Pin = 37.501 Pout = 27.468 Efficiency = 73.2% 5A Power Supply: Load: Efficiency Calculations: Pin = 168.819 Pout = 139.16 Efficiency = 82.4% 10A Power Supply: Load: Efficiency Calculations: Pin = 326.266 Pout = 278.32 Efficiency = 85.3% 12A Power Supply: Load: Efficiency Calculation: Pin = 387.331W Pout = 335.08W Efficiency = 86.5% 12.5A Power Supply: Load: Efficiency Calculation: Pin = 402.550 Pout = 347.90 Efficiency: 86.4% |  |  |
| Action Items: Current output is limited by the DC source power output limit. Find a bigger DC power source. |  |  |
| Test Engineer: LGB, 04/02/26 |  | Witness: GWD, 04/02/26 |

| B 1.3 5VDC Output at 15W |  |  |
| --- | --- | --- |
| Method of Verification: Test |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 3/9/2026 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: 28VDC Power Supply, Electronic load |  |  |
| Test Engineer: Lucas Brickner Witness: Greyson Donley |  |  |
| Criteria for Success: ☒ Passes – Converter delivers 5VDC at 15W ☐ Fails – Converter unable to deliver 5VDC at 15W to the load |  |  |
| Test Procedure: - Set power supply to 28VDC, 1A - Connect the output of the DC supply to the input of the converter - Set the electronic load to 3A - Connect the output of the converter to the DC load - Turn on the power supply - Turn on the electronic load - Record wattage being drawn by the DC load to verify wattage requirement |  |  |
| Test Results: Load: Converter: Supply: Pin = 23.928W Pout = 15.175W Efficiency = 63.4% |  |  |
| Action Items: Passed. No further action. |  |  |
| Test Engineer: LGB, 3/9/26 |  | Witness: GWD, 3/9/26 |

| B 1.4 12VDC Output at 5.4W |  |  |
| --- | --- | --- |
| Method of Verification: Test |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 3/9/2026 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: DC Power Supply, DC electronic load |  |  |
| Test Engineer: Luke Brickner Witness: Greyson Donley |  |  |
| Criteria for Success: ☒ Passes – Converter delivers 12VDC at 5.4W ☐ Fails – Converter unable to deliver 12VDC at 5.4W to the load |  |  |
| Test Procedure: - Set Adjustable DC source to 28V, 1A - Set electronic load to 0.45A - Connect the output of the DC source to the input of the converter - Connect the output of the converter to the DC load - Turn on power supply - Turn on the DC load - Record Wattage being drawn by the DC load to verify wattage requirement |  |  |
| Test Results: Power Supply: Electronic Load: Efficiency Calculation: Pin = 7.662W Pout = 5.419W Efficiency = 70.7% |  |  |
| Action Items: |  |  |
| Test Engineer: LGB, 3/9/26 |  | Witness: GWD, 3/9/26 |

Appendix B2: Software Test Plans

| B 2.1 SPI Code Test |  |  |
| --- | --- | --- |
| Method of Verification: Demonstration |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 3/24/2026 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: Computer, Pico 2, Current Sensor PCB, Micro USB, Power Supply, Electric Load. |  |  |
| Test Engineer: Greyson Donley Witness: Fatima Sayeed |  |  |
| Criteria for Success: ☒ Passes – Current and voltage are displayed in the terminal ☐ Fails – Current and voltage are displayed in the terminal |  |  |
| Test Procedure: - Connect Pico 2 to headers on the Current Sensor PCB. - Connect Pico 2 to Computer using Micro USB. - Retrieve “PICO_SPI” file from Starlab GitHub Page. - Using Visual Studio Code (VS Code), flash code onto Pico 2. - Connect the power supply and load to the sensor PCB based on the schematic “28VDC Sensor Connection”. - Set power supply and load to desired voltage and current to read. - Power Supply current must be set at or above the current setting of the load. - Turn on the load and power supply. - The voltage and current must appear on the VS Code terminal. |  |  |
| Test Results: 2.85V, 8A Voltage: 0%, Current: 0% |  |  |
| Action Items: None. |  |  |
| Test Engineer: GWD, 4/2/2026 |  | Witness: FS, 4/2/2026 |

| B 2.2 I2C Code Test |  |  |
| --- | --- | --- |
| Method of Verification: Demonstration |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 4/7/2026 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: Computer, Raspberry Pi, Pico 2, Current Sensor PCB, Micro USB, Power Supply, Electric Load. |  |  |
| Test Engineer: Fatima Sayeed Witness: Greyson Donley |  |  |
| Criteria for Success: ☒ Passes – Current and Voltage are accurately displayed on to terminal ☐ Fails – Current and Voltage are not accurately displayed on to terminal |  |  |
| Test Procedure: - Connect Pico 2 to headers on the Current Sensor PCB. - Connect Pico 2 and Raspberry Pi pins (SDA to GP6, SCL to GP7, GND to GP3, 3.3V to VSYS). - Connect Pico 2 to Computer using Micro USB. - Retrieve “PICO_SPI” file from Starlab GitHub Page. - Using Visual Code Studio, flash code onto Pico 2. - Connect Pi to Computer using Micro USB. - Configure Raspberry Pi and enable the I2C protocol. - Create a python file in the Raspberry Pi terminal and paste code found in the “PI_MASTER” file (found in Starlab GitHub Page). - Connect the power supply and the load to the PCB based on the schematic “28VDC Sensor Connection” and set to desired current and voltage. - Power Supply current must be set at or above the load’s current setting. - Turn on the load and power supply. - Run the Python file. - The voltage and current must appear on the terminal. |  |  |
| Test Results: 27.748 V, 4.478 A Voltage: 0.063%, Current: 0.49% |  |  |
| Action Items: |  |  |
| Test Engineer: FS, 4/16/2026 |  | Witness: GWD, 4/16/2026 |

Appendix B3: System Test Plans

| B 3.1 Sound Test |  |  |
| --- | --- | --- |
| Method of Verification: Test |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 4/8/2026 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: DC power supply, fan, tape measure, noise measuring tool |  |  |
| Test Engineer: Luke Brickner Witness: Fatima Sayeed |  |  |
| Criteria for Success: ☒ Passes – System is quieter than 64dB at 1m ☐ Fails – System is louder than 64dB at 1m |  |  |
| Test Procedure: - Connected fan to DC power supply - Set power supply to 12V, 0.45A - Measure 1m from the fan and place the sound measuring tool - Turn on the sound measuring tool - Turn on fan through power supply - Record sound being measured in dB to verify sound requirement |  |  |
| Test Results: |  |  |
| Action Items: None |  |  |
| Test Engineer: LGB, 4/8/2026 |  | Witness: FS, 4/8/2026 |

| B 3.2 Size Test |  |  |
| --- | --- | --- |
| Method of Verification: Inspection |  |  |
| Type of Test: First Article, Acceptance |  |  |
| Date(s)/ Total Times: 4/9/2026 | Location: Dreese 761 Capstone Design Lab |  |
| Test Equipment: All components assembled onto mounting plate. Components include 3 buck converters, 2 current sensor PCB’s, Raspberry Pi, and fan. |  |  |
| Test Engineer: Fatima Sayeed Witness: Greyson Donley |  |  |
| Criteria for Success: ☒ Passes – All components are contained within the mounting plate; plate size does not exceed 535 x 600 x 195 mm (or 21 x 23.62 x 7.68 in). ☐ Fails – All components cannot be contained within the mounting plate; plate size exceeds 535 x 600 x 195 mm (or 21 x 23.62 x 7.68 in). |  |  |
| Test Procedure: - Place all components onto the system mounting plate. - Measure length, width, and height. |  |  |
| Test Results: All components fit on plate. Size is 457.2 x 457.2 x 161.93 mm (or 18 x 18 x 6.375 in). The length, width, and height of the system is within range. |  |  |
| Action Items: |  |  |
| Test Engineer: FS, 4/9/2026 |  | Witness: GWD, 4/9/2026 |

Appendix C1: Sensor Accuracy Tests

Current:

| Actual | Measured | Error (%) |
| --- | --- | --- |
| 0.500 | 0.489 | -2.200 |
| 1.499 | 1.487 | -0.801 |
| 2.499 | 2.487 | -0.480 |
| 3.499 | 3.489 | -0.286 |
| 5.499 | 5.477 | -0.400 |
| 6.499 | 6.471 | -0.431 |
| 7.199 | 7.165 | -0.472 |
| Projected |  |  |
| 18.000 | 18.004 | 0.0203 |

Voltage:

| Current | Voltage |  |  |
| --- | --- | --- | --- |
| Actual | Actual | Measured | Error (%) |
| 0.000 | 27.903 | 27.848 | -0.20 |
| 0.500 | 27.886 | 27.838 | -0.17 |
| 1.499 | 27.897 | 27.864 | -0.12 |
| 2.499 | 27.816 | 27.797 | -0.07 |
| 3.499 | 27.780 | 27.777 | -0.01 |
| 5.499 | 27.711 | 27.735 | 0.09 |
| 6.499 | 27.677 | 27.713 | 0.13 |
| 7.199 | 27.650 | 27.697 | 0.17 |
| Projected: |  |  |  |
| 18.000 | 27.367 | 27.560 | 0.72 |

Appendix D: Full Year Gantt Chart